EXP NO: 2

CHARACTERISTICS STUDY OF SIDE BAND POWER AND FREQUENCIES IN AM USING MATLAB

Objectives:

i) To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and modulating signal parameters.

Programming code :

matlab

clc;

clear;

close all;

% Parameters

Am = 1;          % Message signal amplitude

fm = 500;        % Message signal frequency (Hz)

Ac = 5;          % Carrier signal amplitude

fc = 5000;       % Carrier signal frequency (Hz)

fs = 20*fc;      % Sampling frequency

% Time vector

t = 0:1/fs:4/fm;

% Message signal

m = Am*sin(2*pi*fm*t);

% Carrier signal

c = Ac*sin(2*pi*fc*t);

% Modulation index

mu = Am/Ac;

% AM modulated signal (DSB-FC)

am = Ac*(1 + mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

% Demodulation using envelope detection

dem = abs(hilbert(am));

dem = dem - mean(dem);

% FFT calculation

N = length(am);

f = (-N/2:N/2-1)*(fs/N);

AM_FFT = abs(fftshift(fft(am)))/N;

% Sideband frequencies

USB = fc + fm;

LSB = fc - fm;

% Power calculations (assuming 1-ohm load)

Pc = Ac^2/2;

Psb = (mu^2*Pc)/4;

% Display results

fprintf('Modulation Index (mu) = %.2f\n', mu);

fprintf('Upper Sideband (USB) = %d Hz\n', USB);

fprintf('Lower Sideband (LSB) = %d Hz\n', LSB);

fprintf('Carrier Power = %.2f W\n', Pc);

fprintf('Power of Each Sideband = %.3f W\n', Psb);

% Plot results

figure('Name','AM Modulation and Demodulation', ...

'NumberTitle','off');

subplot(3,2,1);

plot(t,m,'b','LineWidth',1.2);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,2);

plot(t,c,'r','LineWidth',1.2);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,3);

plot(t,am,'k','LineWidth',1.2);

title('AM Modulated Signal (DSB-FC)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,4);

plot(t,dem,'g','LineWidth',1.2);

title('Demodulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,2,5);

plot(f,AM_FFT,'LineWidth',1.2);

xlim([fc-3*fm fc+3*fm]);

title('AM Frequency Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

subplot(3,2,6);

bar([Pc Psb Psb]);

set(gca,'XTick',1:3);

set(gca,'XTickLabel',{'Carrier','USB','LSB'});

title('Carrier and Sideband Powers');

ylabel('Power (W)');

grid on;

OUTPUT:

OBJECTIVE:2

ii) To determine and verify the upper sideband (USB) and lower sideband (LSB) frequencies of the AM signal using frequency spectrum analysis in MATLAB.

PROGRAMMING CODE:

clc;

clear;

close all;

% Signal parameters

Am = 1;

fm = 500;

Ac = 5;

fc = 5000;

fs = 20*fc;

% Time vector

t = 0:1/fs:4/fm;

% Generate message and carrier signals

m = Am*sin(2*pi*fm*t);

c = Ac*sin(2*pi*fc*t);

% Generate AM signal

mu = Am/Ac;

am = Ac*(1 + mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t);

% FFT analysis

N = length(am);

Y = fft(am);

P = abs(fftshift(Y))/N;

f = (-floor(N/2):ceil(N/2)-1)*(fs/N);

% Calculate USB and LSB frequencies

USB = fc + fm;

LSB = fc - fm;

% Display results

fprintf('Carrier Frequency = %d Hz\n', fc);

fprintf('Message Frequency = %d Hz\n', fm);

fprintf('Upper Sideband (USB) = %d Hz\n', USB);

fprintf('Lower Sideband (LSB) = %d Hz\n', LSB);

% Plot frequency spectrum

figure;

plot(f, P, 'b', 'LineWidth', 1.2);

xlim([fc-2*fm fc+2*fm]);

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('AM Signal Frequency Spectrum');

grid on;

% Mark carrier, USB, and LSB frequencies

hold on;

xline(LSB, '--r', 'LSB = 4500 Hz');

xline(fc, '--k', 'Carrier = 5000 Hz');

xline(USB, '--g', 'USB = 5500 Hz');

hold off;

OUTPUT:

OBJECTIVE 3:

iii) To calculate the carrier power, sideband power, and total transmitted power of the AM signal for a given modulation index using MATLAB.

PROGRAMMING CODE:

clc;

clear;

close all;

% Given parameters

Ac = 5;           % Carrier amplitude

mu = 0.2;         % Modulation index

R = 1;            % Load resistance in ohms

% Calculate carrier power

Pc = Ac^2/(2*R);

% Calculate sideband powers

P_USB = (mu^2*Pc)/4;

P_LSB = (mu^2*Pc)/4;

% Total sideband power

P_SB = P_USB + P_LSB;

% Total transmitted power

P_total = Pc + P_SB;

% Display results

fprintf('Modulation Index = %.2f\n', mu);

fprintf('Carrier Power = %.3f W\n', Pc);

fprintf('USB Power = %.3f W\n', P_USB);

fprintf('LSB Power = %.3f W\n', P_LSB);

fprintf('Total Sideband Power = %.3f W\n', P_SB);

fprintf('Total Transmitted Power = %.3f W\n', P_total);

% Plot power distribution

figure('Name','AM Power Calculation','NumberTitle','off');

power = [Pc P_USB P_LSB P_total];

labels = {'Carrier','USB','LSB','Total Power'};

bar(power);

set(gca,'XTick',1:4);

set(gca,'XTickLabel',labels);

ylabel('Power (W)');

title('AM Signal Power Distribution');

grid on;

OUTPUT:

OBJECTIVE 4

iv) To analyze the effect of varying the modulation index on sideband power distribution and

overall transmission efficiency through MATLAB simulation.

PROGRAMMING CODE :

clc;

clear;

close all;

% Parameters

Ac = 5;              % Carrier amplitude

R = 1;               % Load resistance in ohms

mu = 0:0.1:1;        % Modulation index from 0 to 1

% Carrier power

Pc = (Ac^2)/(2*R);

% Calculate sideband and total power

P_USB = (mu.^2 * Pc)/4;

P_LSB = (mu.^2 * Pc)/4;

P_SB = P_USB + P_LSB;

P_total = Pc + P_SB;

% Calculate transmission efficiency

efficiency = (P_SB ./ P_total)*100;

% Display results

fprintf(' AM Modulation Index Analysis\n');

fprintf('---------------------------------------------\n');

fprintf(' mu    USB(W)   LSB(W)   Total SB(W)   Total P(W)   Efficiency(%%)\n');

fprintf('---------------------------------------------\n');

for k = 1:length(mu)

fprintf('%.1f   %.3f    %.3f      %.3f         %.3f        %.2f\n', ...

mu(k), P_USB(k), P_LSB(k), P_SB(k), ...

P_total(k), efficiency(k));

end

% Plot 1: Sideband power distribution

figure('Name','Sideband Power Distribution', ...

'NumberTitle','off');

plot(mu, P_USB, '-o', 'LineWidth', 1.5);

hold on;

plot(mu, P_LSB, '-s', 'LineWidth', 1.5);

plot(mu, P_SB, '-^', 'LineWidth', 1.5);

xlabel('Modulation Index (\mu)');

ylabel('Power (W)');

title('Effect of Modulation Index on Sideband Power');

legend('USB Power','LSB Power','Total Sideband Power', ...

'Location','northwest');

grid on;

hold off;

% Plot 2: Total transmitted power

figure('Name','Total Transmitted Power', ...

'NumberTitle','off');

plot(mu, P_total, '-o', 'LineWidth', 1.5);

xlabel('Modulation Index (\mu)');

ylabel('Power (W)');

title('Modulation Index vs Total Transmitted Power');

grid on;

% Plot 3: Transmission efficiency

figure('Name','Transmission Efficiency', ...

'NumberTitle','off');

plot(mu, efficiency, '-o', 'LineWidth', 1.5);

xlabel('Modulation Index (\mu)');

ylabel('Efficiency (%)');

title('Modulation Index vs Transmission Efficiency');

grid on;

OUTPUT: